VENUS 
Motorised gate closer for pedestrian swing gates

CHARACTERISTICS

Electric gate closer.

Features reversible motor.

Can always be operated manually.

Suitable for gates up to max. 100 kg and up to 1400 mm wide.

Maximum motor power of 150 Nm.

For gates with an opening angle of up to 175°.

Opening resistance of maximum 15 Nm.

Closing speed can be adjusted.

Motor power can be adjusted.

Has motor damping.

Has a slow start/stop mode.

Possible to open the gate with a light push (Push-&-Go)

Has integrated obstacle detection.

Has an integrated warning light.

Choice of 3 types of connecting arms:

  - A sliding arm, suitable for combination with 90° hinges

  - An articulated arm, suitable for combination with 90° hinges, with distances of up to 250 mm between motor and hinge shaft

  - An arm to be combined with 180° hinges

Designed and produced in Europe or the United States.

Conforms to ADA (Americans with Disabilities Act) guidelines.

Conforms to ROSPA (British organisation, Royal Society for the Prevention of Accidents) guidelines.

In accordance with the EN12453 & EN16005 standard for the safe use of automatic door systems.

Must be combined with an electric or mechanical lock.

For fully automatic opening, it must be combined with an electromagnetic strike plate, a mechanical lock with electric strike or a motorised bolt.

Available in black or silver.

PERFORMANCE

Specifically developed for outdoor use.

Weatherproof and dustproof.

IP55 rated.

Maintenance-free (no need to lubricate or add oil). Always verify if there are specific safety regulations by local authorities.

2 year manufacturer's warranty.

Tested on 500,000 movements.

INSTALLATION

For gate profiles of at least 80 mm, with a material thickness of at least 2 mm steel

For installation on gate profiles of at least 60 mm or walls, a VENUS-ADAPTERPLATE is available.

To be mounted in drill holes of 15 mm - pre-drilled with a drilling template.

For installation of VENUS-S or VENUS-A on gates with a 180° hinge, a VENUS-RAIL accessory is available that allows the Venus motor to be mounted under the hinge.

Suitable for both left and right turning gates.

Requires a strike to determine the zero position.

No safety sensors required thanks to the standard low energy mode.

3D installation video is available in an app.

3 configurable inputs for connecting access control, security sensors, etc.

3 configurable outputs for connecting an electric strike, electromagnetic strike plate, warning lamp or ambient lamp and a fixed 24V DC power supply.

CONFIGURATION

Programming via an app

Step-by-step guide with video instructions available within the app

App is available for ANDROID and IOS

MATERIAL

Anodised aluminium housing.

Corrosion resistance of 500 hours in salt spray test, according to ISO9227.

UV-resistant and colour-fast.